A Parametric Model For The Calibration And Imaging Costs Of SKA1

Document number	TBD
Revision	C
Author	T.J. Cornwell 
Date	12 Feb 2013
Status	DRAFT

| Name | Designation | Affiliation | Date | Signature |
| --- | --- | --- | --- | --- |
| Additional Authors |  |  |  |  |
| Approved for release: |  |  |  |  |
| P.E. Dewdney | Project Engineer | SKA Organisation |  |  |

DOCUMENT HISTORY

| Revision | Date Of Issue | Engineering Change Number | Comments |
| --- | --- | --- | --- |
| A | 2013Feb07 | - | Converted to SKA format |

DOCUMENT SOFTWARE

|  | Package | Version | Filename |
| --- | --- | --- | --- |
| Wordprocessor | MsWord | Word 2007 | SKA Baseline Computing Costs.docx |

ORGANISATION DETAILS

| Name | SKA Program Development Office |
| --- | --- |
| Physical/Postal Address | SKA Organisation Jodrell Bank Observatory Lower Withington Macclesfield Cheshire SK11 9DL |
| Fax. | +44 (0)161 275 4049 |
| Website | www.skatelescope.org |

TABLE OF CONTENTS

1	Introduction	6

1.1	Purpose of the document	6

1.2	Scope of the document	6

2	References	7

3	Computing Costs	8

4	Pipelines	9

4.1	Ingest pipeline	9

4.2	Calibration pipeline	9

4.3	Continuum imaging	10

4.4	Spectral line imaging	10

4.5	Transient imaging	11

5	Processing Costs	12

5.1	Fresnel number	12

5.2	Measurement size	13

5.3	Image size	13

5.4	A projection	14

5.5	W projection	14

5.6	Kernel initialization	14

5.7	W snapshots	16

5.8	Clean deconvolution	17

5.9	Transient imaging	17

5.10	Scaling philosophy.	17

5.11	Performance And Cost Parametrisation	18

6	SKACost Design Blocks	19

7	Examples	24

8	Accuracy of the model	29

9	Summary	31

10	Acknowledgements	32

11	Example output	33

11.1	Input script	33

11.2	Output	34

LIST OF FIGURES

Figure 1 Generic calibration pipeline	9

Figure 2 Steps in continuum imaging	10

Figure 3 Behaviour of Fresnel number for resolution and field of view. Blue lines show fixed values of the Fresnel number. Green lines show fixed values of the number of pixels. The red lines show values for the SKA1 telescopes. The length of the lines reflects the frequency range. The core consists of the antennas or stations within a radius of 2.5km.	13

Figure 4 SKACost classes implementing this model	19

Figure 6 Processing cost for SKA1_AA_Low as diameter is varied. The sensitivity is held fixed by varying the number of stations in compensation. In this case, the number of beams is increased to maintain an approximately constant field of view. The bumps at the small diameter end are due to beam quantization.	24

Figure 7 Processing costs for SKA1_AA_Low vs array scale.	25

Figure 8 Processing costs for all 250 elements of SKA1_Mid, for Band 1 (350 - 1050MHz).	26

Figure 8 Processing costs for SKA1_Survey, 100 km baseline, vs diameter (10 – 30m)	27

Figure 9 Monte Carlo evaluation of Dish computing cost uncertainty based on factor two up/down in computing cost.	27

Figure 10 Processing costs of SKA1_AA_LOW, 100km baseline, with antenna dependent A matrix, for various intervals between the update of the A kernel.	28

LIST OF TABLES

Table 1 Canonical performance parameters	18

Table 2 Roles of SKACost design blocks	19

Table 3 Inputs parameters for Baseline_{Mid,Survey,AALow}	20

Table 4 Input parameters for Computing_Pipeline. For ease of use, these are also promoted to Baseline_{Mid,Survey,AALow}	20

Table 5 Input parameters for Computing_Vis_Engine	22

Table 6 Input parameters for Computing_Deconvolve_Engine	23

Table 7 Representative costs of SKA1 telescopes. Note the large fraction required for memory. SKA1_AA_Low assumes station-independent A kernels.	31

LIST OF ABBREVIATIONS

AA	Aperture Array

CoDR	Conceptual Design Review

CPU	Central Processing Unit

DRM	Design Reference Mission

EoR	Epoch of Reionisation

FLOPS	Floating Point Operations per second

FoV	Field of View

HPC.............................. High Performance Computer

Hz	Hertz

K	Kelvin

Km	kilometre

PAF	Phased Array Feed

PrepSKA	Preparatory Phase for the SKA

RFI	Radio Frequency Interference

SKA	Square Kilometre Array

SSFoM	Survey Speed Figure of Merit

TBD	To be determined

# Introduction

## Purpose of the document

Computing costs have the potential to be a substantial fraction of the cost for the SKA. These costs include software development costs and calibration and imaging costs. In the past many estimates have been made on the basis of different types and quality of information. In the ASKAP project, there has been a sustained effort to understand the calibration and imaging costs so as to fit within the fixed computing budget. Based on the insights afforded by this effort and by work at LOFAR, in this document we describe a parametric model of the calibration and imaging costs for a wide field of view radio synthesis telescope. The parameterisation is in terms of numbers such as the number of antennas, diameter of antennas, the maximum baseline, the number of beams, and similar. We implement this model using the SKACost[3] costing tool, and illustrate the use of this tool and the model on some possible configurations of SKA1_AA_LOW, SKA1_MID, and SKA1_SURVEY.

## Scope of the document

The model presented in this document covers calibration and imaging processing costs for the three SKA telescopes. The focus is on the capabilities of the telescopes rather than the particular science projects. Hence particular science projects may use considerably fewer resources than described here.

# References

T.J. Cornwell, M. Voronkov, B. Humphreys, “Wide field imaging for the Square Kilometre Array”, arXiv:1207.5861

B. Humphreys and T. Cornwell, SKA Memo 132: Analysis of Convolutional Resampling Algorithm Performance, 2011

D. Ford, R. Bolton, T. Colegate, P. Alexander, P. Hall, SKA Memo 120: The SKA Costing and Design Tool, 2010

Rau, U. & Cornwell, T. J. 2011, A&A, 532, A71

Tasse, C., 2012, Applying full polarization A-Projection to very-wide fields of view instruments: An imager for LOFAR, CALIM 2013.

# Computing Costs

Computing costs for radio synthesis telescopes have been notoriously difficult to estimate. The all too common consequence is that when built the telescope computing is under-resourced. In the case of the SKA, the scale of the computing is so large that rectification of any error could require tens or hundreds of millions of euros. With this in mind, we have initiated a program to develop and update a quantitative, parametric model for computing costs. We expect this program to continue over the years preceding the construction of SKA1 and SKA2.

The computing costs include software development costs and hardware costs for data processing. The data processing includes a number of steps – data acquisition, data transmission, data conditioning, data ingest, calibration and imaging, time series analysis, science analysis, and archive access. From previous analysis, we expect that the calibration and imaging costs for SKA1 will be very large, and so we concentrate on those in this first memo.

This document describes the equations and logic used in estimating the calibration and imaging costs for the three arrays in SKA: SKA1_AA_LOW, SKA1_MID, SKA1_SURVEY.

This document focuses on the processing, memory, and fast storage. The equations are derived for two specific forms of imaging: w projection and w snapshots, and include multi-scale multi-frequency synthesis deconvolution[4].

Future changes or additions to this model are anticipated and should be quite straightforward to implement. We expect that future computer configurations will change the cost and performance numbers encapsulated in the model. We also fully expect that new algorithms will be incorporated. Thus this model should not be seen as definitive but instead as the first of a number of hopefully increasingly accurate models. This work is done in the context of a derivation of a cost cap for SKA1. As the preconstruction phase starts in earnest, the Science Data Processing consortium will undertake refinement of this model.

One last point – this document does not consider particular observations that might be made. Instead it focuses on the capabilities of the telescope. Analysis of specific observations will be presented later.

# Pipelines

At the top level, we will describe the calibration and imaging processing in terms of pipelines performing specific tasks.  These may run autonomously or be invoked by a human. The mode of initiation does not affect the processing load. There is assumed to be a finite computational resource available under the control of SKA and associated with a telescope.

We consider five different pipelines.

- Ingest

- Calibration estimation and application

- Continuum imaging

- Spectral line imaging, including continuum removal

- Transient imaging

When run together, these will satisfy many but not all use cases. We discuss these pipelines in turn.

## Ingest pipeline

The ingest pipeline reads the visibility and meta data from a stream from the telescope and stores them to a fast persistent storage medium. The processing may include flagging for known bad data, editing to remove unknown bad data, calibration, and reformatting.

## Calibration pipeline

The calibration pipeline takes a model of the sky, calculates the visibility resulting for that model and performs a least squares fit to determine antenna gains. We consider here only Direction Independent Effects (DIE). The more difficult case of Direction Dependent Effects (DDE) must be considered in later work. The major work required when calibrating DIE is calculating the model, which is performed only once.

Figure 1 Generic calibration pipeline

Figure 2 Steps in continuum imaging

## Continuum imaging

Continuum imaging constructs an image or set of images of the sky from the visibility data. For high dynamic range imaging, the process is always iterative, as shown in Figure 2. There are two nested iterations, the major cycle that reconciles the current model to the visibility data, and the minor cycle that updates the current model based on a residual image. Although this iteration requires multiple, expensive passes through the data, we currently see no prospect that this can be avoided. A natural consequence is that pure streaming processing will not suffice.

Termination criteria for both levels of iteration are poorly defined and so the number of iterations (minor and major) is difficult to estimate. This is an area where advances in best practice would be particularly helpful.

The channel bandwidth is determined by the requirement to limit bandwidth smearing, and similarly the correlator integration time is determined by the requirement to limit time averaging smearing.

To reach the full dynamic range, multi-frequency synthesis[4] will almost certainly be required. The most commonly used algorithm requires a number of frequency-weighted images to be constructed, at least one for each Taylor term in a frequency expansion of the image. The minor cycle must incorporate joint deconvolution of all the corresponding residual images.

## Spectral line imaging

Prior to imaging the full set of channels an estimate of the continuum must be removed from the visibility. This entails the same model calculation as calibration. Following the removal of the continuum, the full spectral cube can be calculated, involving gridding all the data, Fourier transforming, and then deconvolving. The amount of deconvolution required will depend on the experiment. Generally, we have assumed that the spectral line deconvolution load is insignificant. This assumption warrants careful examination.

## Transient imaging

We consider here slow transient imaging, defined as imaging every correlator integration period. The overall data rate is unaltered so the gridding rate is the same as for continuum imaging. However, an image must be constructed every correlator integration period. A model of the sky is removed from the visibility data before constructing each image. We have assumed that the transient deconvolution load is insignificant. Again, this assumption warrants careful examination.

# Processing Costs

We consider here two contributions to SKA imaging costs – the process of Fourier transforming from image to Fourier space or vice versa, and the process of deconvolution of the point spread function.

Starting with the first process, we use the analysis presented by Cornwell et al.[1]. They discuss two algorithms: w projection and w snapshots. The latter is a hybrid of w projection and snapshot imaging designed specifically to scale more favourably than w projection.

## Fresnel number

The requirement for correction of non-coplanar baselines is determined by the value of the Fresnel number:

If this is less than unity, correction will not be necessary. Otherwise it will be. The desired dynamic range determines the actual dividing line.

In the analysis below, we will treat  and  as the independent variables.

Figure 3 shows the behaviour of the Fresnel number with resolution and field of view, along with the number of pixels on each axis, and the values of the Fresnel number for the SKA1 telescopes.

Figure 3 Behaviour of Fresnel number for resolution and field of view. Blue lines show fixed values of the Fresnel number. Green lines show fixed values of the number of pixels. The red lines show values for the SKA1 telescopes. The length of the lines reflects the frequency range. The core consists of the antennas or stations within a radius of 2.5km.

## Measurement size

An observation will have the following number of visibility samples:

## Image size

To within a factor of order unity related to the sampling and a guard band around the field of view, the number of pixels in the grid or image is determined by the field of view and the resolution:

## A projection

In A projection, the costs are dominated by gridding with the transform of the primary beam (having radius ). The gridding time is:

## W projection

In w projection, the costs are dominated by gridding with the Fresnel kernel. To a first approximation, the widths of the convolution kernels add in quadrature. The computing time is:

where  is the time taken to grid a single sample to a single grid cell,   is the number of visibility samples to be gridded. The scaling numberis roughly the time taken on the chosen computing platform to fetch the data and perform a complex-multiply-accumulate (CMAC). This operation is sufficiently low arithmetic intensity that it is nearly always memory bandwidth limited[2]. The actual performance is therefore limited by factors such as the memory caches, and the memory bandwidth.

Note also the factor . This captures the actual distribution in w of the visibility samples. Ideally this would be computed for each situation. However, the influence on the final results is quite weak and so we typically use a value in the range 0.1 – 0.3.

Once all the data have been gridded, the grid must be Fourier transformed, increasing the time to:

where is the time taken for a 2D complex to complex FFT scaled to a single point.

## Kernel initialization

The W projection kernel must be initialised. The steps involved are:

- Construct image plane by padding by factor e.g. 4 and decimating by same factor e.g. 4

- For each w plane

  - Initialise complex phasor screen in padded, decimated image plane

  - Multiply by gridding correction function

  - FFT to uv space

  - Extract significant part of kernel and discard the rest.

Step 2c is the most time consuming, requiring time:

From Cornwell et al.[1], the number of w planes required is given by:

where  is the allowed amplitude error, and Z is the maximum zenith angle. To within a factor of two, the number of w planes is the Fresnel number. In terms of the field of view and resolution:

For the AW kernel, the illumination pattern is the starting point. Hence the steps are:

- Construct image plane by padding by factor e.g. 4 and decimating by same factor e.g. 4

- For each w plane

  - Initialise illumination pattern in padded, decimated uv space

  - FFT to image space

  - Square to get primary beam

  - Apply complex phasor screen in padded, decimated image plane

  - Multiply by gridding correction function

  - FFT to uv space

  - Extract significant part of kernel and discard the rest.

Hence the number of FFTs is doubled:

In practice, the A kernel may change on a short time scale. For example, for an Aperture Array like LOFAR and SKA1_AA_LOW, even if the telescope is otherwise stable, the projection of the array on the sky changes with azimuth and zenith angle on a time scale . The processing time is then:

This term can easily grow to be large but it still is independent of station. For LOFAR, however, the station beams differ substantially just due to station layout and so the kernels will have to be updated per station, multiplying the work by the number of stations. Finally, it may be necessary to correct non-isoplanaticism via the AW kernel, in which case the kernel may change with time, frequency, and station. Ultimately, the kernel initialization time may come to dominate all other processing[5].

## W snapshots

In the w snapshots algorithm[1], the data are gridded on a uv plane that is oriented in uvw space to be as close as possible to instantaneous uv plane of the array. We can tune by updating the plane whenever the maximum fractional error in w exceeds some limit . Defining:

The cost is then as follows:

We find the optimum value by solving Equation 13 numerically, selecting the root in the range 0,1. There is a minimum value of  allowed. The curvature of the Earth requires that the depth of the convolution function be sufficient to compensate. For 100km baseline, this curvature is 780m. We therefore must clip  to an appropriate minimum value.

For continuum observations, the asymptotic scaling is substantially better than W Projection:

Thus for significant Fresnel number, the time for w snapshots processing has considerably more favorable scaling with field of view and with resolution than w projection. For low Fresnel number, it becomes limited by the A projection gridding rate.

In summary, we have described a tunable approach to gridding and degridding in which the tradeoff between different processing loads can be adjusted dynamically. This is just an extension of the approach presented by Cornwell et al[1]. The payoff is very substantial – the very largest part of the processing may be tuned to a lower level.

Tasse et al.[5] have developed a similar hybrid approach to gridding in which the w projection is implemented partially in image space and partially in uv space. This approach has similar virtues to that described here, and could be modeled in a similar way.

## Clean deconvolution

Once constructed, the images may need to be deconvolved. Here we model the Multi-Scale Multi-Frequency Synthesis CLEAN algorithm[4]. The main work lies in the subtraction of a component from the residual images.

We are using an approximation whereby only pixels in the neighborhood of the peak need be updated. The width of this region is  pixels. This approximation is worthwhile if: .

## Transient imaging

In transient imaging, the entire field of view is imaged every correlator integration time. The image is constructed from the residual visibility resulting from subtracting the model from the observed visibility.

Overall the same amount of gridding is required but many more FFTs are needed. The processing time equation (eqn 13) must be slightly modified:

where  is the number of integrations in the observation.

Since the transient images are constructed from the residual visibility, deconvolution may be barely or not at all necessary.

## Scaling philosophy.

We have expressed the scaling as a linear combination terms of high-level algorithm-focused operations: gridding, image reprojection, kernel initialisation, cleaning. While each of these operations could be decomposed into more compute-centric numbers such as FLOPS and memory bandwidth, there is little extra to be gained and in fact the accuracy of scaling could actually drop.

Thus in this model, a computing facility is characterised by 4 numbers . To the extent that this model is correct, only these numbers need to be benchmarked. However, experience tells us that it is always wise to benchmark the full processing suite when possible.

In addition to processor requirements, we also need to pay attention to the memory requirements. SKA data processing is distinguished by requiring large amounts of memory. This memory problem is made worse by parallelization since by distributing the problem across multiple nodes, we are boosting the memory required above that required by a serial algorithm. Furthermore, inside a node, the memory may or may not be shared amongst all cores. There is no one-size-fits-all solution for this difficulty. We can therefore expect that a significant amount of research will be required to find practical solutions that have low memory costs. In the model presented here, we assume that the processing is distributed by frequency.

## Performance And Cost Parametrisation

According to this model, to evaluate the suitability of a given computing configuration for SKA processing one needs to know the parameters  and the cost of the configuration. We have benchmarked these numbers on a 2012-era supercomputer. These are shown in Table 1. Note that these numbers are scaled from single core or single node.

Table 1 Canonical performance parameters

|  | 8.0e-13 s |
| --- | --- |
|  | 1.1e-11 s |
|  | 9e-10 s |
|  | 1.0e-12s |
| Cost (2012) | €3.2M |

Note that these costs are exclusive i.e. the different operations cannot be performed simultaneously at no extra cost.

Memory costs are difficult to estimate because of the bundled nature of typical procurements. Using the value for a recent large procurement, we find 2012 cost of €50 per GB. For fast storage, we use €610000 per PB.

# SKACost Design Blocks

To explore the consequences of the model, we have used the program SKACost[3]. SKACost represents a telescope as a tree of design blocks (branches) and components (leaves). The components contain a cost value and cost scaling models such as Moore's Law. The design blocks contain logic on how many components are needed for a given set of parameters. The logic is programmed in Python fragments of tens of lines. SKACost has many useful features such as scaling with time, probability distributions for parameters, Monte Carlo evaluations. It can be used from a graphical or command line user interface, or from Python directly. While the GUI is useful for exploration, we use SKACost from Python most of the time mainly to ensure accuracy and repeatability.

The above processing model has been implemented in SKACost design blocks and components. The structure is quite simple (see Figure 4). Note that the same components are reused multiple times.

Figure 4 SKACost classes implementing this model

The roles of the design blocks are shown in Table 2. The input parameters for the blocks are shown in the subsequent tables. Note that a child inherits input parameters from the parent but these are not shown in the table.

Note that all information about the processor is held in the component Computing_Processor. This means that different processors may be evaluated just by changing this component. SKACost makes this easy by making a distinction between interface and implementation.

Finally, the design blocks are structured such that they will fit in as a branch of a broader cost model of the SKA system.

Table 2 Roles of SKACost design blocks

| Baseline_Survey, Baseline_Mid, Baseline_AALow | Design blocks holding definitions of telescopes |
| --- | --- |
| Computing_Pipeline | Design block holding different types of pipeline |
| Computing_Vis_Engine | Design block holding scaling for image construction |
| Computing_Deconvolve_Engine | Design block holding scaling for deconvolution |
| Computing_Storage | Component holding cost of fast storage |
| Computing_Processor | Component holding and cost |
| Computing_Memory | Component holding cost of CPU memory |

Table 3 Inputs parameters for Baseline_{Mid,Survey,AALow}

| Baseline_{Survey,Mid,AALow} |  |
| --- | --- |
| Number_Antennas | Number of antennas in the array. Set to -1 if this is to be calculated from Effective_Diameter and Total_Area (Mid, AALow) or Total_Diameter (Survey). |
| Number_Beams | Number of beams on the sky. |
| Maximum_Baseline | Maximum baseline in array |
| Effective_Diameter | Set to -1 if this is to be calculated from Number_Antennas and Total_Area (Mid, AALow) or Total_Diameter (Survey). |
| Total_Area | The constraint on total area, corresponding to fixed sensitivity. |
| Total_Diameter | The constraint on total diameter, corresponding to fixed survey speed. |

Table 4 Input parameters for Computing_Pipeline. For ease of use, these are also promoted to Baseline_{Mid,Survey,AALow}

| Computing_Pipeline |  |
| --- | --- |
| Bandwidth | The total bandwidth of the observation |
| Data_Retention_Period | The length of time for which the data are kept in (expensive) fast storage |
| Duty_Cycle | The fraction of Observing_Time in which the processing must conclude. Default 1.0 |
| Frequency | The centre frequency of the observation |
| Guard_Band | The total field of view in units of |
| Integration_Time | The integration time in seconds. If set to a negative number, a value will be calculated according to Maximum_Smearing |
| Maximum_Smearing | The maximum tolerated smearing. Not yet well-defined. |
| Number_Channels | The number of channels to be imaged. If set to a value less than unity, this is calculated according to Maximum_Smearing. |
| Number_Continuum_Major_Cycles | The number of major cycles in the continuum image estimation |
| Number_Spectral_Major_Cycles | The number of major cycles in the spectral image estimation |
| Number_Taylor_Terms | The number of Taylor terms used in the expansion of brightness with frequency. |
| Observing_Time | The total observing time. |
| Optimum_rho | controls the tradeoff between w projection and snapshots (see analysis above). If Optimum_rho is set then that value is used. Otherwise, the optimum value is calculated to minimize the processing costs. Set to 1.0 for pure w projection. |
| Oversampling | The number of pixels across the PSF. |
| Pipeline_Type | Type of Pipeline: Ingest; Ingest the visibility and meta data and write to fast storage.. Calibration; Calculate processing of visibility model. Least squares fit and application not yet included.. Continuum; Multi-Scale, Multi-Frequency imaging in the specified number of major cycles.. Spectral_Line; Sky model subtraction followed by deconvolution of each channel, in the specified number of major cycles.. Transient; Sky model subtraction following by imaging for every integration for the specified number of Taylor terms. No deconvolution is performed.. |

Table 5 Input parameters for Computing_Vis_Engine

| Computing_Vis_Engine |  |
| --- | --- |
| Antenna_A | Is the A kernel antenna-dependent? > 0.0 for True |
| A_Time | Interval between recalculations of A matrix |
| Fresnel_Number | (usually need not be specified) |
| Mueller_Multiply | Form of multiplication of visibilities by Mueller matrix: Full (full 16 element), Diagonal (only the diagonal elements) |
| RMS_W | RMS fractional w. Typically 0.1 to 0.3 |
| Number_Images | Number of images to be calculated. Usually 1 but can be much larger for transient processing (usually need not be specified) |

Table 6 Input parameters for Computing_Deconvolve_Engine

| Computing_Deconvolve_Engine |  |
| --- | --- |
| Number_Iterations | Number of minor cycle iterations e.g. clean components found and subtracted. |
| Number_Scales | Number of clean scales to process |
| PSF_Width | Width of the subsection of the PSF used in the deconvolution. Since the SKA PSF is usually very good, this can be less than the full image. |

# Examples

Here we give a number of results from the model. We draw some conclusions in discussion but the main purpose of this section is to show the types of information that can come out of the costing. These examples were all done using simple Python scripts executing SKACost.

The first telescope configuration is a trial SKA1_AA_Low with maximum baseline 100km, 1138 elements each of diameter 53m. All five of the pipeline types are active simultaneously and so the costs simply add. The spectral line pipeline consists only of model subtraction and image construction. The continuum pipeline consists of 10 major cycles cleaning 3 Taylor term images.

In Figure 5, we show the rapid scaling of processing cost downward with increasing station diameter. This cost includes all 5 pipelines.

Figure 5 Processing cost for SKA1_AA_Low as diameter is varied. The sensitivity is held fixed by varying the number of stations in compensation. In this case, the number of beams is increased to maintain an approximately constant field of view. The bumps at the small diameter end are due to beam quantization.

Figure 6 Processing costs for SKA1_AA_Low vs array scale.

The processing costs for SKA_Mid are very large, because of the number of elements and the long baselines involved (see Figure 7) For continuum estimation, the costs can be reduced slightly by using only the antennas on the arms. However, the most effective cost reducing measures are to use a larger diameter dish (continuum processing costs drop a factor of 5 going from 15m to 20m), and to reduce the maximum baseline from 100km to 50km, say.

Figure 7 Processing costs for all 250 elements of SKA1_Mid, for Band 1 (350 - 1050MHz).

In Figure 7, we show the processing of SKA1_Mid versus baseline length. The Calibration, Continuum, and Transient pipelines show scaling as the cube of the baseline, whereas Spectral Line scales roughly as the square. This scaling behaviour is roughly one power of baseline better than that for w projection alone.

Figure 8 Processing costs for SKA1_Survey, 100 km baseline, vs diameter (10 – 30m)

Figure 9 Monte Carlo evaluation of Dish computing cost uncertainty based on factor two up/down in computing cost.

We discussed above the role played by the updates of the AW kernel. In Figure 10, we show the processing costs for SKA1_AA_Low on 100km maximum baseline, with antenna dependent A matrices. The peak cost of €1B far exceeds that for antenna-independent A matrices: €33M. In practice, spectral line processing on the 100km baselines may not be necessary. However, this example does illustrate the importance of considering the stability of the A kernels.

Figure 10 Processing costs of SKA1_AA_LOW, 100km baseline, with antenna dependent A matrix, for various intervals between the update of the A kernel.

# Accuracy of the model

Although the model is detailed, there are many sources of uncertainty:

- We should not be too optimistic about the accuracy of these and any other future cost-performance numbers (see Table 1). Although the performance is measureable, the cost is much more problematic. Moore’s Law has some granularity due to the release of discrete generations of processors. On the contract side, the details of the final contract may not be public and even if then are then it is often the case that the full costs are not itemized with the consequence that distinguishing between processors, memory, interconnect, storage is not feasible. In practice, this means that the cost number in the table about could be a factor of two higher or lower.

- A crucial assumption behind this approach is that the various resources can be used at increasing scale with no loss of efficiency. For example, we implicitly that multiple gridders can be run on a single node with purely linear scale up. In practice, on the latest generation of processors, the efficiency does drop but only to about 75% on a 12 core node.[2]

- The code benchmarked for the numbers in Table 1 is ASKAPSoft, written in C++ and MPI without OpenMP (or similar). Based on a number of lines of evidence, we expect that superior cost/performance can be obtained with either GPUs or Intel Many Integrated Core systems. Hence the performance parameters in Table 1 must be updated regularly as new configurations and code become available.

- The processing costs are dependent on some poorly constrained numbers such as the allowed smearing, the spectral resolution, the required guard band, the number of major cycles, and the duty cycle. These five examples are related to science use cases and the concept of operations. As these aspects of the telescope become better defined, we will be able to improve the accuracy of the cost estimates.

- There are some physical parameters that have a strong effect on the computing costs and are poorly understood. For example, the timescale for antenna-based A matrices, A_Time, drives SKA1_AA_Low processing as the inverse power, for small values (see Figure 10). For a typical value, say 30s, time-variable A matrices could dominate the processing, increasing the costs by up to an order of magnitude. The value is partly dependent on the required dynamic range and is thus amenable to simulations of sufficient fidelity. It is also dependent on timescale for non-isoplanatic ionospheric phase errors across the station beam. This latter timescale will be investigated during LOFAR commissioning. The bottom line is that it is possible that the ionosphere varies in such a way that although it can be estimated, applying the necessary corrections is very expensive.

- The model described here is based on measurements, experience, and understanding from ASKAP in particular. Of the new generation of telescopes, ASKAP is close to unique in having the requirement to process all data automatically in close to real time on a constrained resource. This means that ASKAP has invested significantly in understanding the performance of its pipelines and code. However, ASKAP processing is not necessarily a good model for SKA processing since no deep observations have yet been processed. At the time of writing, LOFAR is both coming into operations and pushing the performance limits. For LOFAR, many of the difficulties now being faced are connected to Direction Dependent Effects, and to the time-variable aspect of the station beams as seen from the sky. Time-variable primary beams are inevitable with Aperture Arrays and the concomitant processing costs probably cannot be avoided, at least for high dynamic range. In addition, for moderate baselines (>10km, say), the ionosphere becomes non-isoplanatic. Solving for and correcting the non-isoplanaticity may introduce substantial computing costs. Both of these effects were easily introduced into our model but we do not yet have enough information to be able to estimate the effect on computing costs.

# Summary

Table 2 shows typical costs for processing and memory for the three telescopes. Note that the numbers here are illustrative and should not be taken as defining actual budgets. The principle means of reducing costs is to change  since the processing costs scale roughly as the cube of this ratio. Note that, for example, the cost of SKA1_Survey can be made comparable to that of SKA1_Mid by reducing the maximum baseline to about 70km.

For context, note that in 2019, €1M will buy roughly 1PF of processing power or about 360TB of memory.

Table 7 Representative costs of SKA1 telescopes. Note the large fraction required for memory. SKA1_AA_Low assumes station-independent A kernels.

| Telescope | Diameter | Baseline | Processor | Memory | Storage | Total |
| --- | --- | --- | --- | --- | --- | --- |
|  | (m) | (km) | (M€) | (M€) | (M€) | (M€) |
| SKA1_AA_Low | 53 | 100 | 6.3 | 1.0 | 1.1 | 8.4 |
| SKA1_Mid | 15 | 100 | 22.6 | 12.2 | 0.8 | 35.6 |
| SKA1_Survey | 15 | 100 | 76.3 | 25.9 | 2.3 | 104.5 |

# Acknowledgements

Much of this work has benefited from discussions with Ben Humphreys. In addition, Ben supplied some of the benchmark numbers used.

I thank Tim Colegate for helpful discussions about SKACost. Dominic Ford and Bojan Nikovic provided access to SKACost source code. SKACost is available via git from:

http://skasim:kl300ui@www.mrao.cam.ac.uk/~bn204/g/SKASim.git/

# Example output

The example shown here is of a Mid configuration. This was generated by SKACost using a custom python script. It explores the uncertainty in cost due to the uncertainty in a number of input parameters: Number_Continuum_Major_Cycles, Number_Spectral_Major_Cycles, Guard_Band, and Number_Scales.

## Input script

'''

Created on 17/01/2013

@author: Tim Cornwell

'''

from skacosthelpers import *

from ska_defs import *

telescope='Mid';variant='';type='Mid';year=2019;curr='EUR'

BaseClasses.cost.Set_DefaultNPVCurr(curr)

BaseClasses.parameter.SetDefaultAccuracy('low')

print "Processing ", telescope+variant

#

# Now read the design blocks and components in the repository

#

r =  BaseClasses.repository.repository()

[warnings] = r.ImportAllScalingRelations(SKArepos)

[components,warnings] = r.ImportAllComponents(SKArepos)

[DesignBlocks,warnings] = r.ImportAllDesignBlocks(SKArepos)

T = BaseClasses.TelescopeDesign.TelescopeDesign(SKArepos+'/SKAcosting2013/Baseline_%s.design' % type)

r.SetCurrentTelescopeDesign(T)

ExtraInputs=SKA[telescope+variant].copy()

print "\n**** Array definition for %s ****" % telescope

print ExtraInputs

for pipeline in ['Ingest', 'Calibration', 'Continuum', 'Spectral_Line', 'Transient', 'All']:

print "\n**** %s pipeline ****\n" % pipeline

ExtraInputs=SKA[telescope+variant].copy()

ExtraInputs['Pipeline_Type']=pipeline

ExtraInputs['Number_Continuum_Major_Cycles']=BaseClasses.parameter.parameter({'min':0, 'max':10}, 'TopHat', DataType='int')

ExtraInputs['Number_Spectral_Major_Cycles']=BaseClasses.parameter.parameter({'min':0, 'max':2}, 'TopHat', DataType='int')

ExtraInputs['Guard_Band']=BaseClasses.parameter.parameter({'min':3, 'max':7}, 'TopHat', DataType='int')

ExtraInputs['Number_Scales']=BaseClasses.parameter.parameter({'min':3, 'max':7}, 'TopHat', DataType='int')

r.FetchDesignBlock('Baseline_%s' % type).build(year, ExtraInputs)

print r.FetchDesignBlock('Computing_Pipeline').StdOut

print "Overall costs for pipeline ", pipeline, r.FetchDesignBlock('Computing_Pipeline').OutputsCumulative['cost']

if pipeline!='Ingest':

print r.FetchDesignBlock('Computing_Vis_Engine').StdOut

print r.FetchDesignBlock('Computing_Deconvolve_Engine').StdOut

print 'Costs: Vis_Engine ', r.FetchDesignBlock('Computing_Vis_Engine').OutputsCumulative['cost'], \

'Deconvolve_Engine ', r.FetchDesignBlock('Computing_Deconvolve_Engine').OutputsCumulative['cost']

print 'Costs: Processor ', r.FetchDesignBlock('Computing_Processor').OutputsCumulative['cost'], \

' Memory ', r.FetchDesignBlock('Computing_Memory').OutputsCumulative['cost'], \

' Storage ', r.FetchDesignBlock('Computing_Storage').OutputsCumulative['cost']

## Output

Processing  Mid

**** Array definition for Mid ****

{'Number_Taylor_Terms': 3, 'Total_Area': 44178.64669110646, 'Mueller_Multiply': 'Diagonal', 'Bandwidth': 700000000.0, 'Number_Continuum_Major_Cycles': 10, 'Duty_Cycle': 1.0, 'Number_Spectral_Major_Cycles': 0, 'Number_Antennas': -1, 'Frequency': 700000000.0, 'Maximum_Baseline': 100000, 'Effective_Diameter': 15}

**** Ingest pipeline ****

Computing_Pipeline Calculating sampling

Computing_Pipeline Observing frequency =  700e6 Hz

Computing_Pipeline Effective diameter of antenna/station =  15 m

Computing_Pipeline Maximum baseline =  100e3 m

Computing_Pipeline Number of antennas/stations =  250

Computing_Pipeline Field of view

Computing_Pipeline 	126.01e-3 +/- 30.91e-3   radians

Computing_Pipeline 	7.09 +/- 1.73   degrees

Computing_Pipeline 	462.47 +/- 102.90   arcmin

Computing_Pipeline 	25.70e3 +/- 6.77e3   arcsec

Computing_Pipeline Resolution

Computing_Pipeline 	4.26e-6   radian

Computing_Pipeline 	243.92e-6   degrees

Computing_Pipeline 	14.63e-3   arcmin

Computing_Pipeline 	878.10e-3   arcsec

Computing_Pipeline 	Number of pixels =  92.60e3 +/- 21.85e3

Computing_Pipeline Fresnel number =  4.12e3 +/- 1.82e3

Computing_Pipeline Continuum processing :  33334  channels of width  24.55e3 +/- 6.47e3 Hz

Computing_Pipeline Integration time =  476.82e-3 +/- 124.49e-3   seconds

Computing_Pipeline Number of transient images =  125664

Computing_Pipeline Spectral Sample rate =  8.50e9 +/- 2.14e9 s^-1

Computing_Pipeline Continuum sample rate =  9.19e9 +/- 2.21e9 s^-1

Computing_Pipeline Data rate from correlator  81.85e-3 +/- 19.44e-3  TB/s for integration time 503.97e-3 +/- 129.11e-3  s

Computing_Pipeline Short term storage required =  6.65 +/- 1.67   PB for  24 hr

Overall costs for pipeline  Ingest 231620.53 +/- 48092.12 EUR (NPV in 2013)

Costs: Processor  0 EUR  Memory  0 EUR  Storage  230263.91 +/- 46363.91 EUR (NPV in 2013)

**** Calibration pipeline ****

Computing_Pipeline Calculating sampling

Computing_Pipeline Observing frequency =  700e6 Hz

Computing_Pipeline Effective diameter of antenna/station =  15 m

Computing_Pipeline Maximum baseline =  100e3 m

Computing_Pipeline Number of antennas/stations =  250

Computing_Pipeline Field of view

Computing_Pipeline 	129.42e-3 +/- 28.19e-3   radians

Computing_Pipeline 	7.43 +/- 1.83   degrees

Computing_Pipeline 	460.51 +/- 101.47   arcmin

Computing_Pipeline 	25.46e3 +/- 6.22e3   arcsec

Computing_Pipeline Resolution

Computing_Pipeline 	4.26e-6   radian

Computing_Pipeline 	243.92e-6   degrees

Computing_Pipeline 	14.63e-3   arcmin

Computing_Pipeline 	878.10e-3   arcsec

Computing_Pipeline 	Number of pixels =  91e3 +/- 22.87e3

Computing_Pipeline Fresnel number =  3.84e3 +/- 1.77e3

Computing_Pipeline Continuum processing :  33334  channels of width  24.66e3 +/- 6.52e3 Hz

Computing_Pipeline Integration time =  500.54e-3 +/- 132.37e-3   seconds

Computing_Pipeline Number of transient images =  83776

Computing_Pipeline Spectral Sample rate =  8.52e9 +/- 2.25e9 s^-1

Computing_Pipeline Continuum sample rate =  9.13e9 +/- 2.19e9 s^-1

Overall costs for pipeline  Calibration 161304.11 +/- 33993.30 EUR (NPV in 2013)

Vis_Engine Assuming Diagonal Mueller matrix multiply

Vis_Engine Finding optimum processing mixture

Vis_Engine 	Approximate Optimum_Rho =  0.00226925398819

Vis_Engine 	Accurate Optimum_Rho    =  1.15e-3

Vis_Engine 	Earth curvature baseline droop = 786.47 m, Minimum rho = 7.86e-3

Vis_Engine 	Clipping optimum rho to minimum

Vis_Engine 	Vis_Engine processing costs (in multiples of processor specified)

Vis_Engine 		 Image FFT  	=  107.80e-6 +/- 49.74e-6

Vis_Engine 		 Evaluating optimum w snapshots settings

Vis_Engine 			 WProject   	=  38.59e3 +/- 43.28e3  		 optimised to  3.19 +/- 2.86

Vis_Engine 			 Kernel     	=  2.20 +/- 1.12  	 optimised to  18.62e-3 +/- 8.85e-3

Vis_Engine 			 Reproject  	=  1.72e-3 +/- 763.48e-6  	 optimised to  203.12e-3 +/- 103.14e-3

Vis_Engine 			 AProject   	=  519.73e-3 +/- 346.82e-3

Vis_Engine 			 Optimum rho	=  7.86e-3

Vis_Engine 		 WSnapshots 		=  3.86 +/- 3.26

Vis_Engine Vis_Engine processing required =  3.45 +/- 3.10

Vis_Engine Vis_Engine memory required     =  1.38 +/- 583.82e-3  TB

Vis_Engine Image disk storage required =  0   TB

Costs: Vis_Engine  167599.02 +/- 34895.55 EUR (NPV in 2013) Deconvolve_Engine  0 EUR

Costs: Processor  165494.79 +/- 31610.94 EUR (NPV in 2013)  Memory  1062.97 EUR (NPV in 2013)  Storage  0 EUR

**** Continuum pipeline ****

Computing_Pipeline Calculating sampling

Computing_Pipeline Observing frequency =  700e6 Hz

Computing_Pipeline Effective diameter of antenna/station =  15 m

Computing_Pipeline Maximum baseline =  100e3 m

Computing_Pipeline Number of antennas/stations =  250

Computing_Pipeline Field of view

Computing_Pipeline 	126.30e-3 +/- 30.14e-3   radians

Computing_Pipeline 	7.46 +/- 1.72   degrees

Computing_Pipeline 	418.56 +/- 111.50   arcmin

Computing_Pipeline 	26.11e3 +/- 6.49e3   arcsec

Computing_Pipeline Resolution

Computing_Pipeline 	4.26e-6   radian

Computing_Pipeline 	243.92e-6   degrees

Computing_Pipeline 	14.63e-3   arcmin

Computing_Pipeline 	878.10e-3   arcsec

Computing_Pipeline 	Number of pixels =  88.40e3 +/- 21.39e3

Computing_Pipeline Fresnel number =  4.35e3 +/- 1.88e3

Computing_Pipeline Continuum processing :  26668  channels of width  24.99e3 +/- 6.65e3 Hz

Computing_Pipeline Integration time =  476.82e-3 +/- 127.77e-3   seconds

Computing_Pipeline Number of transient images =  104720

Computing_Pipeline Spectral Sample rate =  8.88e9 +/- 2.19e9 s^-1

Computing_Pipeline Continuum sample rate =  7.26e9 +/- 1.80e9 s^-1

Overall costs for pipeline  Continuum 1007497 +/- 213845.38 EUR (NPV in 2013)

Vis_Engine Assuming Diagonal Mueller matrix multiply

Vis_Engine Finding optimum processing mixture

Vis_Engine 	Approximate Optimum_Rho =  0.00733336593753

Vis_Engine 	Accurate Optimum_Rho    =  1.35e-3

Vis_Engine 	Earth curvature baseline droop = 786.47 m, Minimum rho = 7.86e-3

Vis_Engine 	Clipping optimum rho to minimum

Vis_Engine 	Vis_Engine processing costs (in multiples of processor specified)

Vis_Engine 		 Image FFT  	=  552.03e-6 +/- 418.39e-6

Vis_Engine 		 Evaluating optimum w snapshots settings

Vis_Engine 			 WProject   	=  33.38e3 +/- 35.85e3  		 optimised to  2.53 +/- 2.20

Vis_Engine 			 Kernel     	=  1.79 +/- 812.75e-3  	 optimised to  14.84e-3 +/- 6.86e-3

Vis_Engine 			 Reproject  	=  10.37e-3 +/- 7.50e-3  	 optimised to  1.32 +/- 899.17e-3

Vis_Engine 			 AProject   	=  2.43 +/- 2.14

Vis_Engine 			 Optimum rho	=  7.86e-3

Vis_Engine 		 WSnapshots 		=  5.51 +/- 4.70

Vis_Engine Vis_Engine processing required =  5.36 +/- 4.20

Vis_Engine Vis_Engine memory required     =  1.33 +/- 636.05e-3  TB

Vis_Engine Image disk storage required =  278.04e-3 +/- 132.03e-3   TB

Deconvolve_Engine Deconvolve_Engine processing required = 164.29e-3 +/- 112.08e-3

Deconvolve_Engine Deconvolve_Engine memory required     =  866.22e-3 +/- 428.14e-3  TB

Costs: Vis_Engine  990337.81 +/- 229895.37 EUR (NPV in 2013) Deconvolve_Engine  341.67 EUR (NPV in 2013)

Costs: Processor  964103.80 +/- 166833.38 EUR (NPV in 2013)  Memory  2733.34 EUR (NPV in 2013)  Storage  9.23 +/- 1.94 EUR (NPV in 2013)

**** Spectral_Line pipeline ****

Computing_Pipeline Calculating sampling

Computing_Pipeline Observing frequency =  700e6 Hz

Computing_Pipeline Effective diameter of antenna/station =  15 m

Computing_Pipeline Maximum baseline =  100e3 m

Computing_Pipeline Number of antennas/stations =  250

Computing_Pipeline Field of view

Computing_Pipeline 	130.27e-3 +/- 31.63e-3   radians

Computing_Pipeline 	7.38 +/- 1.85   degrees

Computing_Pipeline 	440.03 +/- 113.36   arcmin

Computing_Pipeline 	26.34e3 +/- 6.54e3   arcsec

Computing_Pipeline Resolution

Computing_Pipeline 	4.26e-6   radian

Computing_Pipeline 	243.92e-6   degrees

Computing_Pipeline 	14.63e-3   arcmin

Computing_Pipeline 	878.10e-3   arcsec

Computing_Pipeline 	Number of pixels =  84.20e3 +/- 20.45e3

Computing_Pipeline Fresnel number =  3.94e3 +/- 1.88e3

Computing_Pipeline Spectral line processing :  32.77e3   channels

Computing_Pipeline Integration time =  494.69e-3 +/- 125.17e-3   seconds

Computing_Pipeline Number of transient images =  62832

Computing_Pipeline Spectral Sample rate =  9.22e9 +/- 2.33e9 s^-1

Computing_Pipeline Continuum sample rate =  265.58e3 +/- 64.45e3 s^-1

Overall costs for pipeline  Spectral_Line 192295718 +/- 33483032.54 EUR (NPV in 2013)

Vis_Engine Assuming Diagonal Mueller matrix multiply

Vis_Engine Finding optimum processing mixture

Vis_Engine 	Approximate Optimum_Rho =  0.14606368285

Vis_Engine 	Accurate Optimum_Rho    =  44.22e-3

Vis_Engine 	Earth curvature baseline droop = 786.47 m, Minimum rho = 7.86e-3

Vis_Engine 	Vis_Engine processing costs (in multiples of processor specified)

Vis_Engine 		 Image FFT  	=  1.80 +/- 1.09

Vis_Engine 		 Evaluating optimum w snapshots settings

Vis_Engine 			 WProject   	=  22.35e3 +/- 23.71e3  		 optimised to  46.66 +/- 47.07

Vis_Engine 			 Kernel     	=  2.43 +/- 1.07  	 optimised to  95.91e-3 +/- 45.55e-3

Vis_Engine 			 Reproject  	=  29.20 +/- 15.80  	 optimised to  618.24 +/- 329.98

Vis_Engine 			 AProject   	=  239.61e-3 +/- 181.95e-3

Vis_Engine 			 Optimum rho	=  44.22e-3

Vis_Engine 		 WSnapshots 		=  672.85 +/- 408.81

Vis_Engine Vis_Engine processing required =  773.33 +/- 459.10

Vis_Engine Vis_Engine memory required     =  13.59e3 +/- 6.93e3  TB

Vis_Engine Image disk storage required =  3.02e3 +/- 1.51e3   TB

Deconvolve_Engine Deconvolve_Engine processing required = 180.39 +/- 213.72

Deconvolve_Engine Deconvolve_Engine memory required     =  9.37e3 +/- 5.39e3  TB

Costs: Vis_Engine  196444122 +/- 37146327 EUR (NPV in 2013) Deconvolve_Engine  8293152.75 EUR (NPV in 2013)

Costs: Processor  168644531 +/- 31786000 EUR (NPV in 2013)  Memory  26434424.39 EUR (NPV in 2013)  Storage  149470.27 +/- 30498.63 EUR (NPV in 2013)

**** Transient pipeline ****

Computing_Pipeline Calculating sampling

Computing_Pipeline Observing frequency =  700e6 Hz

Computing_Pipeline Effective diameter of antenna/station =  15 m

Computing_Pipeline Maximum baseline =  100e3 m

Computing_Pipeline Number of antennas/stations =  250

Computing_Pipeline Field of view

Computing_Pipeline 	129.70e-3 +/- 32.17e-3   radians

Computing_Pipeline 	7.38 +/- 1.77   degrees

Computing_Pipeline 	430.27 +/- 97.66   arcmin

Computing_Pipeline 	26.93e3 +/- 6.57e3   arcsec

Computing_Pipeline Resolution

Computing_Pipeline 	4.26e-6   radian

Computing_Pipeline 	243.92e-6   degrees

Computing_Pipeline 	14.63e-3   arcmin

Computing_Pipeline 	878.10e-3   arcsec

Computing_Pipeline 	Number of pixels =  90.20e3 +/- 21.45e3

Computing_Pipeline Fresnel number =  4.26e3 +/- 2.05e3

Computing_Pipeline Continuum processing :  26668  channels of width  23.78e3 +/- 5.84e3 Hz

Computing_Pipeline Integration time =  492.29e-3 +/- 119.47e-3   seconds

Computing_Pipeline Number of transient images =  125664

Computing_Pipeline Spectral Sample rate =  9.12e9 +/- 2.36e9 s^-1

Computing_Pipeline Continuum sample rate =  7.42e9 +/- 1.83e9 s^-1

Overall costs for pipeline  Transient 2979474.81 +/- 417512.70 EUR (NPV in 2013)

Vis_Engine Assuming Diagonal Mueller matrix multiply

Vis_Engine Finding optimum processing mixture

Vis_Engine 	Approximate Optimum_Rho =  0.00366668296876

Vis_Engine 	Accurate Optimum_Rho    =  1.87e-3

Vis_Engine 	Earth curvature baseline droop = 786.47 m, Minimum rho = 7.86e-3

Vis_Engine 	Clipping optimum rho to minimum

Vis_Engine 	Vis_Engine processing costs (in multiples of processor specified)

Vis_Engine 		 Image FFT  	=  12.69 +/- 6.53

Vis_Engine 		 Evaluating optimum w snapshots settings

Vis_Engine 			 WProject   	=  39.71e3 +/- 37.18e3  		 optimised to  2.27 +/- 2.19

Vis_Engine 			 Kernel     	=  2.10 +/- 896.18e-3  	 optimised to  15.17e-3 +/- 6.82e-3

Vis_Engine 			 Reproject  	=  1.58e-3 +/- 776.14e-6  	 optimised to  206.42e-3 +/- 99.83e-3

Vis_Engine 			 AProject   	=  409.42e-3 +/- 276.79e-3

Vis_Engine 			 Optimum rho	=  7.86e-3

Vis_Engine 		 WSnapshots 		=  2.85 +/- 2.56

Vis_Engine Vis_Engine processing required =  15.85 +/- 8.41

Vis_Engine Vis_Engine memory required     =  1.37 +/- 591.83e-3  TB

Vis_Engine Transient mode - making 125664 images

Vis_Engine Image disk storage required =  35.50e3 +/- 15.31e3   TB

Costs: Vis_Engine  3016322.61 +/- 439777.99 EUR (NPV in 2013) Deconvolve_Engine  0 EUR

Costs: Processor  1280583.48 +/- 253818.08 EUR (NPV in 2013)  Memory  1062.97 EUR (NPV in 2013)  Storage  1738066.84 +/- 355132.04 EUR (NPV in 2013)

**** All pipeline ****

Computing_Pipeline Calculating sampling

Computing_Pipeline Observing frequency =  700e6 Hz

Computing_Pipeline Effective diameter of antenna/station =  15 m

Computing_Pipeline Maximum baseline =  100e3 m

Computing_Pipeline Number of antennas/stations =  250

Computing_Pipeline Field of view

Computing_Pipeline 	132.82e-3 +/- 30.27e-3   radians

Computing_Pipeline 	7.40 +/- 1.76   degrees

Computing_Pipeline 	451.73 +/- 107.46   arcmin

Computing_Pipeline 	25.87e3 +/- 6.63e3   arcsec

Computing_Pipeline Resolution

Computing_Pipeline 	4.26e-6   radian

Computing_Pipeline 	243.92e-6   degrees

Computing_Pipeline 	14.63e-3   arcmin

Computing_Pipeline 	878.10e-3   arcsec

Computing_Pipeline 	Number of pixels =  93e3 +/- 22.52e3

Computing_Pipeline Fresnel number =  3.90e3 +/- 1.81e3

Computing_Pipeline Continuum processing :  26668  channels of width  24.92e3 +/- 6.38e3 Hz

Computing_Pipeline Integration time =  489.54e-3 +/- 134.04e-3   seconds

Computing_Pipeline Number of transient images =  62832

Computing_Pipeline Spectral Sample rate =  8.82e9 +/- 2.26e9 s^-1

Computing_Pipeline Continuum sample rate =  7.24e9 +/- 1.80e9 s^-1

Computing_Pipeline Data rate from correlator  81.85e-3 +/- 20.73e-3  TB/s for integration time 503.29e-3 +/- 134.69e-3  s

Computing_Pipeline Short term storage required =  7.15 +/- 1.81   PB for  24 hr

Overall costs for pipeline  All 41068826.45 +/- 4645271.93 EUR (NPV in 2013)

Vis_Engine Assuming Diagonal Mueller matrix multiply

Vis_Engine Finding optimum processing mixture

Vis_Engine 	Approximate Optimum_Rho =  0.00366668296876

Vis_Engine 	Accurate Optimum_Rho    =  840.01e-6

Vis_Engine 	Earth curvature baseline droop = 786.47 m, Minimum rho = 7.86e-3

Vis_Engine 	Clipping optimum rho to minimum

Vis_Engine 	Vis_Engine processing costs (in multiples of processor specified)

Vis_Engine 		 Image FFT  	=  118.94e-6 +/- 52.38e-6

Vis_Engine 		 Evaluating optimum w snapshots settings

Vis_Engine 			 WProject   	=  47.07e3 +/- 39.25e3  		 optimised to  2.77 +/- 2.47

Vis_Engine 			 Kernel     	=  2.19 +/- 922.09e-3  	 optimised to  14.24e-3 +/- 7.23e-3

Vis_Engine 			 Reproject  	=  1.77e-3 +/- 795.27e-6  	 optimised to  196.43e-3 +/- 101.19e-3

Vis_Engine 			 AProject   	=  451.30e-3 +/- 286.19e-3

Vis_Engine 			 Optimum rho	=  7.86e-3

Vis_Engine 		 WSnapshots 		=  2.85 +/- 2.55

Vis_Engine Vis_Engine processing required =  2.85 +/- 2.64

Vis_Engine Vis_Engine memory required     =  1.31 +/- 594.17e-3  TB

Vis_Engine Image disk storage required =  0   TB

Vis_Engine Assuming Diagonal Mueller matrix multiply

Vis_Engine Finding optimum processing mixture

Vis_Engine 	Approximate Optimum_Rho =  0.00615964140752

Vis_Engine 	Accurate Optimum_Rho    =  4.07e-3

Vis_Engine 	Earth curvature baseline droop = 786.47 m, Minimum rho = 7.86e-3

Vis_Engine 	Clipping optimum rho to minimum

Vis_Engine 	Vis_Engine processing costs (in multiples of processor specified)

Vis_Engine 		 Image FFT  	=  602.99e-6 +/- 436.90e-6

Vis_Engine 		 Evaluating optimum w snapshots settings

Vis_Engine 			 WProject   	=  37.22e3 +/- 35.90e3  		 optimised to  1.94 +/- 2.07

Vis_Engine 			 Kernel     	=  1.83 +/- 889.63e-3  	 optimised to  14.52e-3 +/- 6.27e-3

Vis_Engine 			 Reproject  	=  9.10e-3 +/- 6.93e-3  	 optimised to  1.23 +/- 857.76e-3

Vis_Engine 			 AProject   	=  1.99 +/- 1.90

Vis_Engine 			 Optimum rho	=  7.86e-3

Vis_Engine 		 WSnapshots 		=  5.79 +/- 5.04

Vis_Engine Vis_Engine processing required =  5.93 +/- 4.97

Vis_Engine Vis_Engine memory required     =  1.36 +/- 654.70e-3  TB

Vis_Engine Image disk storage required =  255.39e-3 +/- 125.95e-3   TB

Vis_Engine Assuming Diagonal Mueller matrix multiply

Vis_Engine Finding optimum processing mixture

Vis_Engine 	Approximate Optimum_Rho =  0.14606368285

Vis_Engine 	Accurate Optimum_Rho    =  110.35e-3

Vis_Engine 	Earth curvature baseline droop = 786.47 m, Minimum rho = 7.86e-3

Vis_Engine 	Vis_Engine processing costs (in multiples of processor specified)

Vis_Engine 		 Image FFT  	=  1.67 +/- 1.04

Vis_Engine 		 Evaluating optimum w snapshots settings

Vis_Engine 			 WProject   	=  22.52e3 +/- 23.10e3  		 optimised to  255.81 +/- 273.86

Vis_Engine 			 Kernel     	=  2.41 +/- 1.13  	 optimised to  275.66e-3 +/- 133.13e-3

Vis_Engine 			 Reproject  	=  26.13 +/- 16.55  	 optimised to  248.15 +/- 134.11

Vis_Engine 			 AProject   	=  288.34e-3 +/- 213.77e-3

Vis_Engine 			 Optimum rho	=  110.35e-3

Vis_Engine 		 WSnapshots 		=  537.45 +/- 443.31

Vis_Engine Vis_Engine processing required =  540.15 +/- 442.38

Vis_Engine Vis_Engine memory required     =  15.44e3 +/- 6.44e3  TB

Vis_Engine Image disk storage required =  3.04e3 +/- 1.43e3   TB

Vis_Engine Assuming Diagonal Mueller matrix multiply

Vis_Engine Finding optimum processing mixture

Vis_Engine 	Approximate Optimum_Rho =  0.00488891062502

Vis_Engine 	Accurate Optimum_Rho    =  1.03e-3

Vis_Engine 	Earth curvature baseline droop = 786.47 m, Minimum rho = 7.86e-3

Vis_Engine 	Clipping optimum rho to minimum

Vis_Engine 	Vis_Engine processing costs (in multiples of processor specified)

Vis_Engine 		 Image FFT  	=  7.02 +/- 3.18

Vis_Engine 		 Evaluating optimum w snapshots settings

Vis_Engine 			 WProject   	=  37.22e3 +/- 35.90e3  		 optimised to  2.47 +/- 2.38

Vis_Engine 			 Kernel     	=  1.93 +/- 954.83e-3  	 optimised to  15.28e-3 +/- 6.98e-3

Vis_Engine 			 Reproject  	=  1.87e-3 +/- 824.12e-6  	 optimised to  220.20e-3 +/- 97.48e-3

Vis_Engine 			 AProject   	=  428.97e-3 +/- 252.73e-3

Vis_Engine 			 Optimum rho	=  7.86e-3

Vis_Engine 		 WSnapshots 		=  3.39 +/- 2.80

Vis_Engine Vis_Engine processing required =  10.14 +/- 6.23

Vis_Engine Vis_Engine memory required     =  1.27 +/- 593.62e-3  TB

Vis_Engine Transient mode - making 62832 images

Vis_Engine Image disk storage required =  17.06e3 +/- 8.32e3   TB

Deconvolve_Engine Deconvolve_Engine processing required = 186.38e-3 +/- 115.57e-3

Deconvolve_Engine Deconvolve_Engine memory required     =  892.41e-3 +/- 480.21e-3  TB

Deconvolve_Engine Deconvolve_Engine processing required = 202.72 +/- 206.70

Deconvolve_Engine Deconvolve_Engine memory required     =  8.87e3 +/- 4.78e3  TB

Costs: Vis_Engine  36778500 +/- 4553510.62 EUR (NPV in 2013) Deconvolve_Engine  4687377.67 +/- 3875.88 EUR (NPV in 2013)

Costs: Processor  23413247 +/- 5046879.84 EUR (NPV in 2013)  Memory  16282582.19 EUR (NPV in 2013)  Storage  1026300.12 +/- 199664.77 EUR (NPV in 2013)